# STM32H747I-DISCO — SW Block Diagram & Memory Placement Audit

# STM32H747I-DISCO — SW Block Diagram & Memory Placement Audit

Generated: 2026-05-11 Source of truth: EWARM/STM32H747I-DISCO_CM7/List/STM32H747I-DISCO_CM7.map Linker config: EWARM/stm32h747xx_flash_CM7.icf Active branch: test/inject-flasher-thumbnail-no-wifiThis is example for a project using STM32H747 device from STmicroelectronic, the project involved few periperals: Touchscreen, UART, DFSDM (Voice microphone ),  Filesystem that writes to SD card.

When working with the STM32H7, the primary logic to consider is the distributed memory and bus architecture. Unlike simpler MCUs where memory is a single uniform block, the H7 is split into distinct domains (D1, D2, and D3) connected by a complex Bus Matrix. To prevent system stalls or "jitter," a developer must surgically place data based on its consumer: high-speed application code and graphics should live in the D1-domain AXI SRAM for maximum CPU performance, while peripheral data (like UART or SAI buffers) should ideally reside in D2-domain SRAM1/2 to allow the DMA controllers to move data without competing for the CPU’s local buses. Ignoring this "spatial logic" leads to bus contention, where a high-speed peripheral can effectively starve the CPU, causing the real-time deadlocks often seen in complex multitasking projects.

The second critical consideration is the management of Cache Coherency and the MPU (Memory Protection Unit). Because the Cortex-M7 uses a high-speed L1 cache to achieve its 480 MHz performance, the data the CPU "sees" in its cache may not match the actual data a DMA controller is writing to physical RAM. Logic dictates that any memory region used for DMA must either be configured as "Non-Cacheable" via the MPU or requires the developer to manually perform Cache Maintenance (Clean/Invalidate) operations. Failing to synchronize these two "views" of memory results in corrupted data packets—such as missing UART bytes or distorted audio—which are notoriously difficult to debug because they appear as random software glitches rather than the hardware configuration errors they truly are.

## 1. Memory Domains (STM32H747 silicon)

┌──────────────────────────────────────────────────────────────────────────────┐
│  D1 — CORE DOMAIN (always-on while M7 runs)                                  │
│  ────────────────────────────────────────────────                            │
│  CPU:    Cortex-M7 @ 400 MHz                                                 │
│  DMA:    MDMA (all domains), DMA2D (D1 only)                                 │
│  Bus:    AXI / AHB1 / AHB3                                                   │
│  RAM:                                                                        │
│    • Flash      0x08000000  (1 MB)     code, .intvec, const tables           │
│    • ITCM       0x00000000  (64 KB)    UNUSED (no DMA reach)                 │
│    • DTCM       0x20000000  (128 KB)   CSTACK (8 KB), HEAP (1 KB) - NO DMA   │
│    • AXI SRAM   0x24000000  (512 KB)   ALL FreeRTOS tasks, framebuffer,      │
│                                          heap_4 (96 KB), HAL handles,        │
│                                          UART/queue/mutex CBs                │
└──────────────────────────────────────────────────────────────────────────────┘
                        │ AHB crossbar (CM7 can reach D2/D3 RAM)
┌──────────────────────────────────────────────────────────────────────────────┐
│  D2 — PERIPHERAL DOMAIN                                                      │
│  ────────────────────────────────                                            │
│  CPU:    Cortex-M4 @ 200 MHz (NOT used — CM4 stub only at 0x08100000)        │
│  DMA:    DMA1 (audio: DFSDM→SRAM1), DMA2 (SD: SDMMC1 IDMA bounce)            │
│  Bus:    AHB / APB1 / APB2                                                   │
│  Peripherals: DFSDM1, SDMMC1, SAI1-3, UART8, USART1-3, SPI, I2C              │
│  RAM:                                                                        │
│    • SRAM1   0x30000000  (128 KB)   s_DfsdmBuf @ 0x30004000  [512 B DMA ring]│
│    • SRAM2   0x30020000  (128 KB)   g_AudioBuf @ 0x30020000  [96 KB PCM]     │
│    • SRAM3   0x30040000  (32 KB)    UNUSED                                   │
│  Reach: DMA1/DMA2 can access D1 AXI + D2 SRAM (NOT D3)                       │
└──────────────────────────────────────────────────────────────────────────────┘
                        │ internal silicon bridge (SAI4→DFSDM1 — N/A on this branch)
┌──────────────────────────────────────────────────────────────────────────────┐
│  D3 — LOW-POWER DOMAIN                                                       │
│  ────────────────────────                                                    │
│  CPU:    none                                                                │
│  DMA:    BDMA — ONLY reaches D3 SRAM4                                        │
│  Peripherals: SAI4 (gated off — Path C-PC2 uses DFSDM-master),               │
│               LPTIM, RTC, LPUART1, ADC3, DAC, I2C4, SPI6                     │
│  RAM:                                                                        │
│    • SRAM4   0x38000000  (64 KB)    UNUSED in this project                   │
│    • Backup  0x38800000  (4 KB)     UNUSED                                   │
└──────────────────────────────────────────────────────────────────────────────┘

## 2. SW Component Block Diagram (audio pipeline + UART bridge)

╔══════════════════════════════════════════════════════════════════════════════╗
║                          STM32H747 CM7 — VOICE RECORDER                      ║
╚══════════════════════════════════════════════════════════════════════════════╝

   ┌──── HARDWARE ─────┐                  ┌──── HARDWARE ─────┐
   │  MP34DT05-A mic   │                  │  NORA-W106 ESP32  │
   │  PDM data on PC1  │                  │  UART8 @ 921600   │
   │  CLK from PC2     │                  │  Wi-Fi → GCS/STT  │
   └─────┬─────────────┘                  └────────┬──────────┘
         │ PDM 2 MHz                               │ UART8 TX/RX
         ▼                                         ▼
   ┌──────────────────┐                  ┌──────────────────────┐
   │ DFSDM1_Channel0  │                  │      UART8 HAL       │
   │  (clock master + │                  │  (DMA1_Stream0 RX,   │
   │   data sampler)  │                  │   blocking TX)       │
   └─────┬────────────┘                  └────────┬─────────────┘
         │ Sinc3, OSR=125                         │ ISR + DMA-IDLE
         │ → 16 kHz PCM24                         │
         ▼                                        ▼
   ┌────────────────────────────┐       ┌────────────────────────────┐
   │     DMA1_Stream1           │       │     DMA1_Stream0           │
   │  (DFSDM1_Filter0 → SRAM1)  │       │  (UART8_RX → AXI SRAM)     │
   │  s_DfsdmBuf @ 0x30004000   │       │  s_dma_rx_buf @ 0x2406EEC0 │
   │  128 × int32, circular     │       │  128 bytes, normal mode    │
   │  MPU: Non-Cacheable,Share  │       │  D-Cache enabled (invalid. │
   │  D2 SRAM1                  │       │  on RX complete)           │
   └─────────────┬──────────────┘       └─────────────┬──────────────┘
                 │ DMA HT/TC ISR                      │ IDLE ISR fires
                 ▼                                    ▼
   ┌────────────────────────────┐       ┌────────────────────────────┐
   │  HAL_DFSDM_Filter…Cplt-     │       │ HAL_UARTEx_RxEventCallback │
   │   Callback (ISR)            │       │  (UART8_IRQHandler)        │
   │  → store via StoreDmaChunk  │       │  → SCB_InvalidateDCache    │
   │     (CPU >>8 shift)         │       │  → memcpy to BleRawMsg_t   │
   └─────────────┬──────────────┘       │  → xQueueSendFromISR       │
                 │ writes int16          │     into xRawBleQueue      │
                 ▼                       └─────────────┬──────────────┘
   ┌────────────────────────────┐                     │ 4-slot queue
   │   g_AudioBuf @ 0x30020000  │                     ▼
   │   48000 × int16 (96 KB)    │       ┌────────────────────────────┐
   │   D2 SRAM2                 │       │   xRawBleQueue (static)    │
   │   MPU: Non-Cacheable,Share │       │   Handle  @ 0x2406FBDC     │
   └─────────────┬──────────────┘       │   CB      @ 0x2406F2C4     │
                 │                       │   Storage @ 0x2407ACB4     │
                 │                       │   4 × BleRawMsg_t (520 B)  │
                 │                       │   AXI SRAM                 │
                 │                       └─────────────┬──────────────┘
                 │                                     │
                 ▼                                     ▼
╔══════════════════════════════ FreeRTOS TASKS ════════════════════════════════╗

   ┌──────────────────────┐      ┌──────────────────────┐
   │  VoiceRecTask        │      │  UARTReceiveTask     │
   │  Priority 32         │      │  Priority 26         │
   │  Stack 3072 W (AXI)  │      │  Stack 1024 W (AXI)  │
   │                      │      │                      │
   │  - Drain DFSDM DMA   │      │  - Recv from queue   │
   │  - Run 3-sec window  │      │  - Parse AUDIO:READY │
   │  - Signal SDWrite    │      │  - Notify SDWrite    │
   └──────────┬───────────┘      └──────────┬───────────┘
              │ xVoiceQueue                 │ task notify
              ▼                             ▼
   ┌────────────────────────────────────────────────────┐
   │  SDWriteTask  (Priority 20, Stack 2048 W in AXI)   │
   │  - Open REC_XXX.wav, write header                  │
   │  - Bounce g_AudioBuf via s_pcmBounce (AXI)         │
   │  - SDMMC1 IDMA writes to SD card                   │
   │  - Then: send "AUDIO:FILE name SIZE" via UART8 TX  │
   │  - Wait for "AUDIO:READY" notification             │
   │  - Stream 96 KB PCM over UART8 TX                  │
   └──────────────────┬─────────────────────────────────┘
                      │
                      ▼
   ┌──────────────────────┐      ┌──────────────────────┐
   │  VoiceCMDhandler     │      │  RTTLogTask          │
   │  Priority 16         │      │  Priority 1          │
   │  Stack 1536 W (AXI)  │      │  Stack 1024 W (AXI)  │
   │  - Process commands  │      │  - Drain xLogQueue   │
   │    from xRawBleQueue │      │  - Print to RTT + IO │
   └──────────────────────┘      └──────────────────────┘

## 3. Memory Placement Audit (current `.map`)

### D1 AXI SRAM `0x24000000 .. 0x2407FFFF` — 512 KB

| Range | Size | Symbol / Owner | Domain | Notes |
| --- | --- | --- | --- | --- |
| 0x24000000 – 0x24046500 | 281 KB | TouchGFX_Framebuffer (PFB strip) | D1 AXI | FB_BLOCK, 32-aligned, LTDC scan target |
| 0x24046500 – 0x24046734 | ~564 B | .data (init data) | D1 AXI | RTT, OS wrappers, locale data |
| 0x24046734 – 0x240467AC | 120 B | s_hdma_uart8_rx (DMA handle) | D1 AXI | ble_uart.o |
| 0x240467AC – 0x24050000 | ~38 KB | TouchGFX engine, JPEG, BSP, FatFS .bss | D1 AXI | task globals |
| 0x24050000 – 0x24050800 | 2 KB | g_irq_log (IRQ ring, 256×8 B) | D1 AXI | stm32h7xx_it.o |
| 0x24050800 – 0x24050804 | 4 B | g_irq_idx | D1 AXI | running count |
| 0x24050804 – 0x24068EA4 | 96 KB | `ucHeap` (FreeRTOS heap_4 default) | D1 AXI | heap_4 internal xHeap |
| 0x24068EA4 – 0x2406EEC0 | ~24 KB | rtos_trace bufs + misc | D1 AXI |  |
| 0x2406EEC0 – 0x2406EF40 | 128 B | `s_dma_rx_buf` ⭐ | D1 AXI | UART8 RX DMA target |
| 0x2406EFBC – 0x2406F034 | 120 B | s_hdma_dfsdm | D1 AXI | DFSDM DMA handle |
| 0x2406F034 – 0x2406F0AC | 120 B | hdma_sai4_a_rx | D1 AXI | (unused but linked) |
| 0x2406F25C – 0x2406F2C4 | 104 B | hdma2d + misc HAL handles | D1 AXI |  |
| 0x2406F2C4 – 0x2406F324 | 96 B | `s_rawBleQueueCB` ⭐ NEW | D1 AXI | StaticQueue_t |
| 0x2406F324 – 0x2406F384 | 96 B | `s_bleHistMutexCB` ⭐ NEW | D1 AXI | StaticSemaphore_t |
| 0x2406F55C – 0x2406F844 | ~744 B | hdfsdm1_filter0, hdfsdm1_channel0 | D1 AXI | DFSDM HAL handles |
| 0x2406FBD8 – 0x2406FC88 | 176 B | xBleHistMutex, xRawBleQueue, xLogQueue, xVoiceQueue, xMusicQueue, g_pfb_dbg_* | D1 AXI | queue/mutex handles + debug counters |
| 0x2407ACB4 – 0x2407AEBC | 520 B | `s_rawBleQueueStorage` ⭐ NEW | D1 AXI | 4 × BleRawMsg_t (132 B each) |
| 0x2407C2C0 – 0x2407C4C0 | 512 B | UARTReceiveTask::accum | D1 AXI | UART line accumulator |
| 0x2407C640 – 0x2407C780 | 320 B | bleHistory (10×32) | D1 AXI | UI history grid |
| 0x2407C960 – 0x2407C9F4 | 148 B | huart8 (HAL handle) | D1 AXI |  |
| 0x2407CDE8 – 0x2407FFFF | ~13 KB | FREE | — | unused tail |

⭐ = new symbols introduced 2026-05-11 (static-queue migration + section attribution).

### D1 DTCM `0x20000000 .. 0x2001FFFF` — 128 KB

| Range | Size | Symbol | Domain | Notes |
| --- | --- | --- | --- | --- |
| 0x20000000 – 0x20002000 | 8 KB | CSTACK (M7 main + ISR stack) | D1 DTCM | NO DMA reach |
| 0x20002000 – 0x20003000 | 4 KB | HEAP (C lib heap, unused) | D1 DTCM | NO DMA reach |
| 0x20003000 – 0x2001FFFF | ~116 KB | FREE | — | unused |

### D2 SRAM1 `0x30000000 .. 0x3001FFFF` — 128 KB

| Range | Size | Symbol | Domain | Notes |
| --- | --- | --- | --- | --- |
| 0x30000000 – 0x30004000 | 16 KB | FREE | — | unused |
| 0x30004000 – 0x30004200 | 512 B | `s_DfsdmBuf` | D2 SRAM1 | DFSDM1 DMA ring (#pragma location) |
| 0x30004200 – 0x3001FFFF | ~112 KB | FREE | — | unused |

### D2 SRAM2 `0x30020000 .. 0x3003FFFF` — 128 KB

| Range | Size | Symbol | Domain | Notes |
| --- | --- | --- | --- | --- |
| 0x30020000 – 0x30037700 | 96 KB | `g_AudioBuf` | D2 SRAM2 | 48000 × int16 PCM, #pragma location |
| 0x30037700 – 0x3003FFFF | ~34 KB | FREE | — | unused |

### D3 SRAM4 `0x38000000 .. 0x3800FFFF` — 64 KB

| Range | Owner | Notes |
| --- | --- | --- |
| ENTIRE 64 KB | UNUSED | SAI4 retired in Path C-PC2; BDMA not used |

### External SDRAM `0xD0000000 .. 0xD1FFFFFF` — 32 MB

| Range | Size | Symbol | Domain | Notes |
| --- | --- | --- | --- | --- |
| 0xD0000000 – 0xD02A3000 | 2.6 MB | s_rgb888Buf | SDRAM | JPEG decode intermediate |
| 0xD02A3000 – 0xD0465000 | 1.7 MB | s_ycbcrBuf | SDRAM | JPEG MCU output |
| 0xD0465000 – 0xD057E400 | 1.1 MB | s_rgb888Scaled | SDRAM | scaled 800×480 RGB888 |
| 0xD05B0400 – 0xD1FFFFFF | ~26 MB | FREE | — | unused tail |

## 4. Collision Audit

### Check 1 — Audio buffers in their own domain

| Buffer | Address | Domain | DMA controller | Status |
| --- | --- | --- | --- | --- |
| s_DfsdmBuf | 0x30004000 | D2 SRAM1 | DMA1_Stream1 | ✓ DMA1 can write D2 SRAM1 |
| g_AudioBuf | 0x30020000 | D2 SRAM2 | CPU writes only | ✓ Reachable from CPU |
| Gap between them | 124 KB | — | — | ✓ No overlap, no neighbor |

### Check 2 — UART RX pipeline same domain

| Buffer | Address | Domain | Path | Status |
| --- | --- | --- | --- | --- |
| s_dma_rx_buf | 0x2406EEC0 | D1 AXI | DMA1_Stream0 RX target | ✓ DMA1 can write D1 AXI |
| s_rawBleQueueCB | 0x2406F2C4 | D1 AXI | ISR xQueueSendFromISR writes | ✓ Same domain as DMA buf |
| s_rawBleQueueStorage | 0x2407ACB4 | D1 AXI | 4-slot ring buffer | ✓ Same domain |
| s_bleHistMutexCB | 0x2406F324 | D1 AXI | Mutex CB | ✓ CPU-only access |

No cross-domain bus traffic per RX message.

### Check 3 — Framebuffer isolated from audio

| Buffer | Address | Domain | Master | Status |
| --- | --- | --- | --- | --- |
| TouchGFX_Framebuffer (PFB) | 0x24000000 | D1 AXI | LTDC | ✓ |
| s_DfsdmBuf | 0x30004000 | D2 SRAM1 | DMA1 | ✓ Different domain — no bus contention |
| g_AudioBuf | 0x30020000 | D2 SRAM2 | CPU | ✓ Different domain |

LTDC scans AXI; audio lives in D2. Eliminates FUIF bandwidth contention.

### Check 4 — FreeRTOS heap vs DFSDM DMA buffer

| Item | Address | Domain | Notes |
| --- | --- | --- | --- |
| ucHeap (heap_4) | 0x24050804, 96 KB | D1 AXI | ✓ Different domain from DFSDM |
| s_DfsdmBuf | 0x30004000, 512 B | D2 SRAM1 | ✓ Isolated |

Yesterday's bisect concern (heap @ 0x30004200 adjacent to DFSDM buf) is resolved — heap is now in AXI, far away.

### Check 5 — Stack vs DMA buffers

| Item | Range | Notes |
| --- | --- | --- |
| CSTACK (M7) | 0x20000000–0x20002000 (DTCM) | ✓ DMA cannot reach DTCM — safe by design |
| Task stacks | inside AXI .bss | ✓ Each task has private stack guarded by FreeRTOS |

### Check 6 — Free space margins

| Domain | Used | Free | Margin |
| --- | --- | --- | --- |
| D1 AXI SRAM | ~499 KB | ~13 KB | 2.5% headroom — tight but OK |
| D1 DTCM | 12 KB | 116 KB | 90% free |
| D2 SRAM1 | 0.5 KB | 127.5 KB | 99% free |
| D2 SRAM2 | 96 KB | 32 KB | 25% free |
| D3 SRAM4 | 0 | 64 KB | 100% free |
| SDRAM | ~5.4 MB | ~26 MB | 81% free |

AXI SRAM is the tightest — if a future feature needs >13 KB additional .bss in D1, move it to D2 SRAM1 or SRAM2 via #pragma location = ".sram1" / ".sram2" (sections defined in .icf).

## 5. Hard Rules — Where Each Buffer Class Belongs

| Buffer class | Required region | Why |
| --- | --- | --- |
| LTDC framebuffer | D1 AXI SRAM (0x24000000) | LTDC scans on AHB; AXI removes contention with FMC/SDRAM |
| DFSDM DMA ring | D2 SRAM1 (0x30000000+) | DMA1 lives in D2; placing in D2 SRAM avoids AXI contention with LTDC |
| PCM accumulator (`g_AudioBuf`) | D2 SRAM2 (0x30020000+) | CPU writes only, large (96 KB) — keeps D1 AXI free for framebuffer |
| SDMMC1 IDMA bounce | D1 AXI SRAM | SDMMC1 IDMA prefers AXI; bounce buffer copied from D2 |
| UART8 RX DMA buf | D1 AXI SRAM | DMA1/2 reachable; same domain as ISR-driven queue send |
| FreeRTOS queues / mutexes / task TCBs | D1 AXI SRAM (default) | CPU-only access; keep with task stacks |
| FreeRTOS heap (ucHeap) | D1 AXI SRAM | Default; never place adjacent to DMA ring buffers |
| CSTACK + ISR stack | DTCM (0x20000000) | Zero-wait, NO DMA can corrupt it |
| JPEG decode intermediates | External SDRAM | Multi-MB, latency-tolerant |
| TouchGFX assets | QSPI Flash (XIP) / SDRAM | Read-only LUTs at runtime |
| SAI4 BDMA buffers (if ever used) | D3 SRAM4 (0x38000000) | BDMA is D3-only; never place SAI4-BDMA in D1/D2 |

## 6. Section Attribution Cheat Sheet (Hard Rule #3)

/* Default .bss/.data → D1 AXI SRAM (no attribute needed) */
static uint32_t g_counter;

/* Explicit AXI placement (rare, but documents intent) */
#pragma location = ".axi_sram"
static StaticQueue_t s_queueCB;

/* D2 SRAM1 — DMA1/DMA2 reachable */
#pragma location = ".sram1"
static uint8_t s_dmaBuf[256];

/* D2 SRAM2 — DMA1/DMA2 reachable */
#pragma location = ".sram2"
static int16_t s_audioBlock[8192];

/* Absolute address (bypass section regions) — for hardware-fixed buffers */
#pragma location = 0x30004000
static __no_init uint32_t s_DfsdmBuf[128];

## 8. References

EWARM/stm32h747xx_flash_CM7.icf — linker script (memory regions + section placements)

EWARM/STM32H747I-DISCO_CM7/List/STM32H747I-DISCO_CM7.map — authoritative symbol locations

CLAUDE.md Hard Rule #1 (no dynamic allocation) + Hard Rule #3 (section attribution)

ST AN4861 (LTDC), AN4891 (system topology), AN5215 (bus bandwidth), AN5405 (cache & coherency)

RM0399 §2 (memory map), §60.5 (DBGMCU freeze bits)

## 9.  Rules for Efficient STM32H7 Application Architecture

- 1. Implement a "Fast-Path" AXI Domain Strategy

- The STM32H7 bus matrix is complex; for maximum efficiency, you must treat memory domains as specialized zones:

- AXI SRAM (D1 Domain): Reserve this for the "Fast-Path"—CPU-intensive tasks and high-bandwidth data. All RTOS objects (Task Stacks, Mutexes, Queues) and graphics framebuffers should live here to ensure the CPU can access them with minimal latency.

- DTCM (Data Tightly Coupled Memory): Use this exclusively for the main system stack (CSTACK). It provides zero-wait-state access, which is critical for high-speed interrupt processing and critical sections.

- SRAM1/2 (D2 Domain): Dedicate this domain to "Peripheral-Only" DMA buffers (e.g., DFSDM, SAI, Ethernet). By keeping these buffers in D2, their DMA traffic does not compete with the CPU or Graphics traffic in the AXI domain.

- 2. Enforce Strict Static Allocation

- Avoid dynamic memory allocation (malloc, pvPortMalloc) in high-performance firmware.

- Predictability: Static allocation ensures memory addresses are fixed at link-time, preventing non-deterministic behavior during runtime.

- Fragmentation: Eliminates the risk of memory fragmentation, which is vital for long-running systems at 480 MHz.

- Debugging: Static objects have fixed addresses in the .map file, making it possible to use C-SPY macros to inspect them without firmware instrumentation.

- 3. Use Mandatory IAR Placement (Section Attribution)

- To prevent the Linker from accidentally placing a high-speed queue in a slow memory region, explicitly define placements:

- Use #pragma location = ".axi_sram" or the @ syntax for CPU-bound data.

- Use .sram1_data for DMA-bound buffers.

- The Rule: Never let critical buffers "float" in the default .bss or .data sections; always architect their physical location.

- 4. Minimize Cross-Domain Bus Traffic

- Collisions often occur when a DMA controller in one domain tries to access memory in another domain while the CPU is busy.

- Isolation: Keep the UART DMA and its corresponding processing queues in the same domain (AXI).

- Independence: Moving the FreeRTOS heap out of the D2 domain (where audio DMA lives) prevents task-creation logic from stalling the audio pipeline.

- 5. Adopt Zero-Footprint Debugging (ITM & C-SPY)

- When debugging timing-sensitive issues, avoid printf or heavy logging.

- Hardware Tracing (ITM): Use the Instrumentation Trace Macrocell for nanosecond-accurate, hardware-timestamped events with zero CPU overhead.

- Headless Diagnostics: Use cspybat with .mac files to perform "X-ray" inspections of peripheral registers without modifying or halting the firmware. This preserves the real-time 60Hz sync of your multitasking environment.

- 6. Cautions: Cache Coherency and MPU

- MPU Configuration: Always configure MPU regions for DMA buffers. Mark SRAM regions used by DMA as "Bufferable/Non-Cacheable" to avoid data corruption between the CPU cache and the DMA controller.

- Priority Inversion: In multitasking environments, always use Mutexes for shared resources (to benefit from priority inheritance) and Semaphores only for signaling/synchronization.

Best Architect Setup for Efficiency

| Component | Memory Region | Address Range (Approx) | Why? |
| --- | --- | --- | --- |
| Main Stack | DTCM | 0x20000000 | Zero-wait-state CPU access. |
| Graphics FB | AXI SRAM | 0x24000000 | High-bandwidth D1 domain access. |
| RTOS Queues | AXI SRAM | 0x2406xxxx | Fast CPU processing; isolated from D2 DMA. |
| Audio Buffers | SRAM1 (D2) | 0x3000xxxx | Dedicated peripheral DMA lane. |
| Heap | AXI SRAM | 0x2405xxxx | Prevents "Pathfinder" stalls in peripheral domains. |